Clinical Rotation Site:__________________________

Preceptor Printed Name(s):

___________________________

Chemistry Clinical Rotation

Competency Checklist

Student Name: _______________________

| Area | Specific requirements | Notes | Check off |
| --- | --- | --- | --- |
| Quality Control and Maintenance | Performs QC on routinely used chemistry analyzers and immunoassays |  | ⃝ Completed ⃝ Not Applicable |
|  | Observes or performs preventative maintenance on all chemistry analyzers |  |  |
|  | Evaluates Levey-Jennings charts for Westgard Rule violations. |  |  |
|  | Troubleshoot QC violations on all chemistry analyzers |  |  |
| Calibrations and linearity | Discuss/observe calibration procedure and protocol of different analytes. |  | ⃝ Completed ⃝ Not Applicable |
|  | Discuss/observe frequency and procedure for linearity studies of chemistry equipment |  |  |
| Specimen Acceptability | Discuss the reasons and follow-up procedures for rejection of samples according to department protocol. |  | ⃝ Completed ⃝ Not Applicable |
|  | Determines appropriate sample type (serum/plasma, additive, etc.) |  |  |
|  | Identifies preanalytical errors and/or physiological conditions which will interfere with specific tests, ie, hemolysis, lipemia, icterus, incorrect tube additive, etc. |  |  |
| Analyzer operation of Patient Samples and Reporting | Read/discuss principle of the instrument operation or test procedure reaction. |  | ⃝ Completed ⃝ Not Applicable |
|  | Perform instrument start-up and/or shutdown. |  |  |
|  | Appropriately acts on results beyond the linearity and/or reportable range of the instrument/network |  |  |
|  | Discuss “critical” or “panic” values and reporting protocol |  |  |
|  | Discuss and observe LIS result entry. |  |  |
|  | Discuss and observe delta check protocol. |  |  |
|  | Discuss and perform dilutions when necessary. |  |  |
| Referral (send out) Testing | Discuss/observe specimen processing, paperwork, and tests involved in referral testing |  | ⃝ Completed ⃝ Not Applicable |
| Theory | Discuss the clinical significance of abnormal results obtained, correlating patient results as to possible disease and/or therapy states. |  | ⃝ Completed ⃝ Not Applicable |
| Special Procedures | Osmometry | ⃝ Completed ⃝ Not Available | Other/Notes: |
|  | Serology (Flu, Strep, HCG, Mono, etc.) | ⃝ Completed ⃝ Not Available |  |
|  | Electrophoresis | ⃝ Completed ⃝ Not Available |  |
|  | Sweat Chloride | ⃝ Completed ⃝ Not Available |  |
|  | Esoteric chemistry testing: |  |  |